SONY ST-SA3ES

ESP32 RDS Interface Reference

Experimental decoding, protocol baseline and injection design

Purpose Document the verified RDS clock/data interface between the SAA6579T demodulator and the Sony controller, preserve the decoded evidence, and define the baseline required to implement an ESP32-generated RDS stream for the original Sony display.

| Item | Value |
| --- | --- |
| Status | Experimentally decoded at the RDS group level |
| Verified sample | 5965 consecutive samples from the 20 kHz capture |
| Station PI | 0x89FF |
| Verified PS evidence | Block D = 0x464D = "FM" |
| Verified RadioText evidence | Blocks C/D = 0x4D2C 0x2056 = "M, V" |
| Primary next step | ESP32 replay of valid captured groups through a safe 3.3 V-to-5 V interface |

Decision on more capture data No additional capture is required to establish this implementation baseline. The current sample proves valid RDS CRC framing, PI 0x89FF, a Group 0A PS segment and a Group 2A RadioText segment. A second non-overlapping sample is recommended later to validate all four PS segments, more RadioText positions and repetition behaviour before final firmware acceptance.

# 1. Scope and design boundary

The goal is not to generate an FM multiplex signal or a 57 kHz RDS subcarrier. The Sony tuner already uses an SAA6579T demodulator. That device outputs recovered RDS data and a recovered RDS clock for processing by the Sony controller. The ESP32 implementation will target this post-demodulator interface.

Core design rule Generate the same recovered clock/data stream that the SAA6579T presents to the Sony controller. Do not implement FM modulation, 57 kHz subcarrier generation, biphase decoding or differential decoding in the injection path.

This note complements the existing FM frequency-control baseline. That separate interface lets the ESP32 observe the frequency requested by the original controller, while the RDS interface provides a route for presenting station names and stream metadata through the Sony controller and original display.

# 2. Physical interface

| Connector | Pin | Sony label | RDS role |
| --- | --- | --- | --- |
| CN701 | 5 | RDS-C | Recovered RDS clock input to the Sony controller |
| CN702 | 6 | RDS-D | Recovered RDS data input to the Sony controller |

Logic-analyser capture mapping: the first exported column is RDS-D and the second exported column is RDS-C. The clock column toggles regularly while the data column remains stable across the sampling transition.

| Source signal | SAA6579T pin | Function |
| --- | --- | --- |
| RDDA | 2 | RDS data output |
| RDCL | 16 | RDS clock output |

Electrical caution The Sony logic was measured at approximately 4.8 V. Do not connect the Sony RDS outputs directly to ESP32 GPIO pins. For active ESP32 injection, isolate or select away the SAA6579T outputs and use a 3.3 V-to-5 V logic buffer. Avoid two active outputs driving the same line.

# 3. Capture and extraction method

| Parameter | Value | Assessment |
| --- | --- | --- |
| Sample rate | 20 kHz | Sufficient oversampling for the recovered clock |
| Original capture | 500,000 samples | 25 seconds total |
| Decoded test subset | 5,965 samples | Approximately 0.298 seconds |
| Recovered clock edges | 354 | 354 recovered data bits |
| Data polarity | Normal | Inverted data did not produce valid complete groups |
| Column order | RDS-D, RDS-C | Confirmed by edge regularity and CRC-valid groups |

The decoder tested both columns as possible clock/data assignments, both rising and falling clock edges, and normal/inverted data. Only column 1 as data, column 2 as clock, and normal polarity produced repeated complete A-B-C-D RDS groups. Rising and falling edge extraction produced the same decoded groups because the data was stable across each clock transition in the examined sample.

For each clock transition: bit = read(RDS_D) shift bit into a 26-bit window calculate CRC syndrome using polynomial 0x5B9 compare syndrome with RDS offset words synchronize blocks as A -> B -> C/C' -> D

# 4. RDS framing baseline

A complete RDS group contains four 26-bit blocks. Every block contains 16 information bits followed by a 10-bit checkword. The offset word identifies the block position and supports synchronization.

| Block | Offset syndrome | Primary content |
| --- | --- | --- |
| A | 0x0FC | Programme Identification (PI) |
| B | 0x198 | Group type, version and control fields |
| C | 0x168 | Group-dependent data |
| C' | 0x350 | Alternative Block C for version B groups |
| D | 0x1B4 | Group-dependent data, including PS or RadioText characters |

Decoder constants Generator polynomial: 0x5B9. Complete group length: 104 bits. The implementation must produce valid checkwords and offset words; sending plain ASCII characters without valid RDS framing will not be accepted as a normal RDS stream.

# 5. Experimentally decoded groups

Two complete, CRC-valid groups were recovered from the 5,965-sample subset. Both carried PI 0x89FF, matching the known station information.

## 5.1 Group 0A: Programme Service evidence

| Block | Information word | Decoded meaning |
| --- | --- | --- |
| A | 0x89FF | PI = 89FF |
| B | 0x042F | Group type 0A; PS segment address 3 |
| C | 0xCACD | Group 0A application data |
| D | 0x464D | ASCII 0x46 0x4D = "FM" |

Expected eight-character PS name: VELUWEFM Segment 0: VE Segment 1: LU Segment 2: WE Segment 3: FM <- directly observed as Block D 0x464D

This does not by itself recover all eight characters from the examined subset. It does directly verify the final PS segment, and that segment is consistent with the expected station name VELUWEFM.

## 5.2 Group 2A: RadioText evidence

| Block | Information word | Decoded meaning |
| --- | --- | --- |
| A | 0x89FF | PI = 89FF |
| B | 0x2422 | Group type 2A and RadioText segment fields |
| C | 0x4D2C | ASCII 0x4D 0x2C = "M," |
| D | 0x2056 | ASCII 0x20 0x56 = " V" |

Recovered four-character RadioText fragment: "M, V" Expected text: "Veluwe FM, Voor Harderwijk, Ermelo en Putten" ^^^^ observed fragment fits this boundary

The recovered fragment is positioned consistently with the known RadioText. The sample does not establish the entire RadioText string, but it proves that standard Group 2A text data is present on the interface.

# 6. What is verified, supported and still open

| Classification | Item |
| --- | --- |
| Verified | CN701 pin 5 carries RDS-C and CN702 pin 6 carries RDS-D. |
| Verified | The first capture column is data and the second capture column is clock. |
| Verified | Normal data polarity produces complete CRC-valid RDS groups. |
| Verified | PI 0x89FF appears in both decoded Block A words. |
| Verified | A Group 0A Block D carries 0x464D, the characters "FM". |
| Verified | A Group 2A carries the characters "M, V" across Blocks C and D. |
| Supported interpretation | The Sony controller consumes standard post-demodulator RDS groups from the SAA6579T interface. |
| Open | Exact Sony sampling edge. Both edges decoded identically in this subset because data remained stable. |
| Open | Complete recovery of PS segments VE, LU and WE from additional non-overlapping data. |
| Open | Complete RadioText assembly, A/B toggle behaviour and text update timing. |
| Open | Whether the Sony controller requires QUAL or other status behaviour during active injection. |
| Open | Final line isolation/multiplexer and level-shifter circuit. |

# 7. ESP32 injection architecture

Internet-radio metadata ¦ v RDS group builder: 0A (PS) + 2A (RadioText) ¦ v CRC-10 and A/B/C/D offset-word encoder ¦ v 104-bit group scheduler ¦ v 1187.5 bit/s data output + recovered-style clock output ¦ v 3.3 V to 5 V buffer / source selector ¦ v Sony RDS-D and RDS-C inputs -> Sony controller -> original display

## 7.1 Hardware boundary

| Requirement | Baseline |
| --- | --- |
| Isolation | The SAA6579T and ESP32 must not drive the Sony inputs simultaneously. |
| Level translation | Convert ESP32 3.3 V outputs to the Sony 5 V logic domain. |
| Ground | Use an appropriate common digital reference. |
| Selection | Provide an original-RDS / ESP32-RDS source selection mechanism. |
| Initial prototype | Prefer a reversible, current-limited bench arrangement before permanent PCB integration. |

## 7.2 Firmware modules

| Module | Responsibility |
| --- | --- |
| Metadata state | PI, PTY, PS, RadioText and update flags |
| Group 0A builder | Emit four two-character PS segments |
| Group 2A builder | Emit four RadioText characters per segment |
| CRC/checkword encoder | Generate 10-bit checkword and apply block offset |
| Scheduler | Repeat PS more frequently and cycle RadioText segments |
| Bit output engine | Present data and clock with stable setup/hold timing |
| Validation logger | Record generated groups and compare against offline decoding |

## 7.3 Recommended first firmware milestone

Replay before generation First replay the two verified captured groups using ESP32-generated clock and data. If the Sony controller accepts the replay, the electrical interface, polarity, bit order and timing are proven. Only then replace captured groups with dynamically generated 0A and 2A groups.

Verified group 0A information words: 89FF 042F CACD 464D Verified group 2A information words: 89FF 2422 4D2C 2056 Important: calculate and append each block's 10-bit checkword before transmission.

# 8. Validation plan

| Test | Method | Pass condition |
| --- | --- | --- |
| Offline encoder | Generate known 0A/2A groups and run them through the same decoder | PI, group type and character fields decode exactly |
| Captured-group replay | Transmit the two verified groups to the isolated Sony RDS inputs | Sony controller accepts the stream without bus contention |
| PS assembly | Cycle VE, LU, WE, FM Group 0A segments | Display reconstructs VELUWEFM |
| RadioText assembly | Cycle Group 2A segments for a controlled short string | Displayed text matches the controlled string |
| Metadata update | Change RadioText and toggle the RDS text A/B flag | Sony refreshes rather than mixing old and new text |
| Electrical test | Scope or analyse both buffered outputs under load | Levels and edge timing remain within the intended logic domains |
| Fallback | Switch source back to the SAA6579T | Original FM RDS operation returns cleanly |

# 9. Additional capture recommendation

More data is useful for confidence, but it is not required before writing the first ESP32 replay and encoder prototype. Before calling the implementation production-ready, decode a second non-overlapping capture containing enough repeated groups to recover all four PS segments and a larger section of RadioText.

| Priority | Capture objective | Reason |
| --- | --- | --- |
| High | Recover VE, LU, WE and FM in valid 0A groups | Confirms segment addressing and full PS assembly |
| High | Recover consecutive 2A segments | Confirms RadioText addressing and ordering |
| Medium | Observe a RadioText change | Confirms A/B toggle and receiver refresh behaviour |
| Medium | Capture loss and reacquisition of RDS | Determines whether another status line or timeout behaviour matters |
| Low | Capture a station with clock-time data | Only required if time display will be emulated |

Engineering judgement Proceed with the reference document and first ESP32 implementation now. Request another 5,000-to-10,000-line non-overlapping chunk when validating full PS/RadioText assembly, not as a prerequisite for starting development.

# 10. Reference decoder logic

The offline decoder used for the experiment should remain part of the project. It gives the ESP32 implementation a deterministic regression test: generated bitstreams can be sampled or exported and decoded using the same CRC and group parser.

offsets = {A:0x0FC, B:0x198, C:0x168, Cprime:0x350, D:0x1B4} polynomial = 0x5B9 for each selected RDS-C edge: append RDS-D to bitstream for each 26-bit sliding window: syndrome = polynomial_remainder(window) if syndrome matches an offset: record block type and 16-bit information word accept a complete group only when blocks occur at 26-bit spacing: A -> B -> C or Cprime -> D

# 11. References

Sony project FM baseline: Sony_ST-SA3ES_ESP32_FM_Frequency_Control.docx, internal project document.

Experimental sample: 5969lines.txt, 5,965 consecutive samples from RDS-20Khz500.

Full capture: RDS-20Khz500.csv / .log, 20 kHz, 500,000 samples.

SAA6579T data sheet: https://media.digikey.com/pdf/Data%20Sheets/NXP%20PDFs/SAA6579.pdf

RDS encoder reference: https://github.com/hayguen/mpxgen/blob/master/src/rds.c

ESP32 RDS assembler reference: https://github.com/MarcFinns/PiratESP32-FM-RDS-STEREO-ENCODER/blob/main/RDSAssembler.h